CPE301 – FALL 2019

# Design Assignment 2A

Student Name: Andrew Buchanan

Student #:5003154346

Student Email: buchaa2@unlv.nevada.edu

Primary Github address: https://github.com/buchaa2/103EPC

Directory: https://github.com/buchaa2/103EPC/tree/master/DA2A

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

List of Components used

Block diagram with pins used in the Atmega328P

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

; DA2A.asm

;

; Created: 9/28/2019 1:18:05 AM

; Author : ANDREW BUCHANAN

;

; Replace with your application code

; GPIOs_Part1_ASM.asm

.org 0

sbi DDRB, 3		; set PORTB.3 as output

sbi PORTB, 3	; set LED off

; generate waveform with 40% DC and .625 sec period

BEGIN:

sbi PORTB, 3	; set LED on

rcall delay_1

cbi PORTB, 3	; set LED off

rcall delay_2

jmp BEGIN

; Delay 4 000 000 cycles

; 250ms at 16 MHz

delay_1:

ldi  r18, 21

ldi  r19, 75

ldi  r20, 191

L1: dec  r20

brne L1

dec  r19

brne L1

dec  r18

brne L1

nop

ret

; Delay 6 000 000 cycles

; 375ms at 16 MHz

delay_2:

ldi  r18, 31

ldi  r19, 113

ldi  r20, 31

L2: dec  r20

brne L1

dec  r19

brne L1

dec  r18

brne L2

nop

ret

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

int main(void)

{

DDRB |= (1<<3);		// set PORTB.3 for output

while (1)

{

_delay_ms(250);		// delay for 40% DC

PORTB &= ~(1<<3);	// make port B LED high

_delay_ms(375);		// the rest of the .650ms delay

PORTB |= (1<<3);	// make port B LED low

}

}

; DA2A_part2.asm

;

; Created: 9/30/2019 9:22:22 PM

; Author : ANDREW BUCHANAN

;

; GPIOs_Part2_ASM.asm

.org 0

cbi	DDRC, 3		; set PORTC.3 to be an input

sbi	PORTC, 3	; set PORTC.3 to be active high

sbi	DDRB, 2	    ; set PORTB.3 to be an output output

sbi PORTB, 2	; set LED off

FUNC:

sbic PINC, 3	; if PINC is low go to line 17

jmp	SKIP		; jump to pin C

sbi PORTB, 2	; turn LED on

; Delay 21 328 000 cycles

; 1s 333ms at 16 MHz

ldi  r18, 109

ldi  r19, 51

ldi  r20, 106

L1: dec  r20

brne L1

dec  r19

brne L1

dec  r18

brne L1

jmp FUNC

SKIP:

Cbi	PORTB, 2	; turn LED off

jmp FUNC

; DA2A_part2.asm

;

; Created: 9/30/2019 9:22:22 PM

; Author : ANDREW BUCHANAN

;

; GPIOs_Part2_ASM.asm

.org 0

cbi	DDRC, 3		; set PORTC.3 to be an input

sbi	PORTC, 3	; set PORTC.3 to be active high

sbi	DDRB, 2	    ; set PORTB.3 to be an output output

sbi PORTB, 2	; set LED off

FUNC:

sbic PINC, 3	; if PINC is low go to line 17

jmp	SKIP		; jump to pin C

sbi PORTB, 2	; turn LED on

; Delay 21 328 000 cycles

; 1s 333ms at 16 MHz

ldi  r18, 109

ldi  r19, 51

ldi  r20, 106

L1: dec  r20

brne L1

dec  r19

brne L1

dec  r18

brne L1

jmp FUNC

SKIP:

Cbi	PORTB, 2	; turn LED off

jmp FUNC

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

Part 1 C code

Part 2

Part2 C code

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

- GITHUB LINK OF THIS DA

https://github.com/buchaa2/103EPC/tree/master/DA2A

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Andrew Buchanan