CPE301 – FALL 2019

# Design Assignment 2B

Student Name: Andrew Buchanan

Student #:5003154346

Student Email: buchaa2@unlv.nevada.edu

Primary Github address: https://github.com/buchaa2/103EPC

Directory:

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

;

- ; DA2B.asm

- ;

- ; Created: 10/5/2019 9:15:12 PM

- ; Author : bugul

- ;

- .org 0			; location of the reset

- jmp MAIN		; start the main program

- .org 0x02		; location for external interrupt INT0

- jmp EXT_INT		; start the external interrupt

- MAIN:

- ldi R20, HIGH(RAMEND)	; initialize stack

- out SPH, R20

- ldi R20, LOW(RAMEND)

- out SPL, R20

- ldi R20, 2		; make INT0 falling edge triggered

- sts EICRA, R20

- sbi DDRB, 3		; set PORTB.2 as output

- sbi PORTB, 3	; start the PORTB.3 LED off

- sbi PORTD, 3	; turn on PORTD.3

- ldi R20, 1<<INT0	; enable INT0

- out EIMSK, R20

- sei				; set the interupt

- WHILE:

- sei				; set the interupt again

- ; generate waveform with 40% DC and .625 sec period

- cbi PORTB, 3	; set LED on

- ; Delay 4 000 000 cycles

- ; 250ms at 16 MHz

- ldi  r18, 21

- ldi  r19, 75

- ldi  r20, 191

- L1: dec  r20

- brne L1

- dec  r19

- brne L1

- dec  r18

- brne L1

- nop

- cbi PORTD, 3	; turn on pull-up

- sbi PORTB, 3	; set LED off

- ; Delay 6 000 000 cycles

- ; 375ms at 16 MHz

- ldi  r18, 31

- ldi  r19, 113

- ldi  r20, 31

- L2: dec  r20

- brne L2

- dec  r19

- brne L2

- dec  r18

- brne L2

- nop

- jmp	WHILE		; wait for interrupt

- EXT_INT:

- cbi PORTB, 3

- ; Delay 17 328 000 cycles

- ; 1s 83ms at 16 MHz

- ldi  r18, 88

- ldi  r19, 232

- ldi  r20, 170

- L3: dec  r20

- brne L3

- dec  r19

- brne L3

- dec  r18

- brne L3

- jmp WHILE	; reti was taking a really long time to return to the while loop so i just

- ; made the interupt jump back up to the while loop then set the inerupt again

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://www.youtube.com/watch?v=Xu2gUbivu08

- GITHUB LINK OF THIS DA

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Andrew Buchanan