CPE301 – FALL 2019

# Design Assignment 2C

Student Name: Andrew Buchanan

Student #:5003154346

Student Email: buchaa2@unlv.nevada.edu

Primary Github address: https://github.com/buchaa2/103EPC

Directory:

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

//DA2A task1

#define F_CPU 16000000UL

#include <avr/io.h>

int main(void)

{

DDRB =0xFF;		// set PORTB registers as outputs for output (LED)

DDRC =0x00;		// set PORTC registers as inputs  for input switches

PORTB =0xFF;		// turn all the LEDs in B on

TCCR0A = 0;			// normal operation

TCCR0B |= (1<<CS02) | (1<<CS00); // prescaler = 1024

int count = 0;		// overflow counter

while (1) {

count = 0;					// reset counter

TCNT0 = 0x00;				// reset timer

while (count < 15){			// loop until 40% DC is met

if(TCNT0 == 0xFF){		// if timer overflows

count++;			// increment counter

TCNT0 = 0x00;		// reset timer

}

}

PORTB =0x00 ;			// toggle LED off

count = 0;					// reset counter

TCNT0 = 0x00;				// reset timer

while (count < 23){			// loop until remaining 60% DC is met

if(TCNT0 == 0xFF){		// if timer overflows

count++;			// increment counter

TCNT0 = 0x00;		// reset timer

}

}

PORTB =0xFF;			// toggle LED on

}

}

-----------------------------------------------------------------------------------------

//DA2A task2

#define F_CPU 16000000UL

#include <avr/io.h>

int main(void)

{

DDRC &= (0<<3);		// connect PORTC.3 to switch as input

PORTC |= (1<<3);	// enable pull-up

DDRB = 0xFF;		// set PORTB.2 for output (LED)

TCCR0A = 0;			// normal operation

TCCR0B |= (1<<CS02) | (1<<CS00); // prescaler = 1024

int count = 0;		// overflow counter

while (1) {

if(!(PINC & (1<<PINC3))) {	// check if pin C 3 is low

PORTB =0x00;		// set LED on

count = 0;				// reset counter

TCNT0 = 0x00;			// reset timer

while(count < 82) {		// loop until 1.33 sec delay met

if(TCNT0 == 0xFF){	// if timer overflows

count++;		// increment counter

TCNT0 = 0x00;	// reset timer

}

}

}

else {

PORTB = 0xFF;		// set LED off

}

}

return 0;

}

-----------------------------------------------------------------------------------------

//DA2A task 1

#include <avr/io.h>

#include <stdio.h>

#include <avr/interrupt.h>

//declare vital variables

int ovFlowCount = 0;

int dc60 = 23; //overflow count to reach is 23

int dc40 = 15; //overflow count to reach is 15

int flag = 1;

int main(void)

{

DDRB |= (1<<2)|(1<<5);		// Set all PORTB out

PORTB |= (1<<5);			// Set PORTB

TCCR0B |= (1<<CS00)|(1<<CS02);	// Set pre-scaler to 1024

TIMSK0 |= (1<<TOIE0); //enable timer interrupts

sei(); //enable interrupts

TCNT0 = 0; //initialize clock

while(1);

}

ISR(TIMER0_OVF_vect){

ovFlowCount ++;

if (flag == 1 && ovFlowCount == dc40){

PORTB &= ~(1<<2);

ovFlowCount = 0;	//reset overflow counter

flag = 0;

}

else if (flag == 0 && ovFlowCount == dc60){

PORTB |= (1<<2);

ovFlowCount = 0;	//reset overflow counter

flag = 1;

}

}

-----------------------------------------------------------------------------------------

/DA2A task2

#include <avr/io.h>

#include <stdio.h>

#include <avr/interrupt.h>

int flag = 0;

//FOR 1.333s Overflow count = 82

int main(void){

//initialize registers

DDRB |= (1<<2)|(1<<5);		//set portb 2 to output

PORTB |= (1<<2)|(1<<5);		//set pinb2 to high

DDRC &= (0<<2);		//set ddrc to input

PORTC |= (0<<2);		//set portc2 to low

TCCR0B |= (1<<CS00)|(1<<CS02);	// Set pre-scaler to 1024

//int flag = 0;

TIMSK0 |= (1<<TOIE0);		//enable timer interrupt

sei();			//enable interrupt

while (1) {

if (!(PINC & (1 << PINC2))) //check for button press

{

flag = 1;		//if pressed then turn on led

}

else {

PORTB |= (1<<2);		//if not pressed then keep led off

flag = 0;		//keep flag off to keep from false positives

}

}

return 0;

}

ISR(TIMER0_OVF_vect){

int ovFlowCount = 0;

int ovCheck = 0;

int delayCount = 82;		//amount of overflows to reach desired delay

if (flag == 1){

PORTB &= ~(1<<2);		//turn LED on

TCNT0 = 0;				//start counter at 0

while(ovFlowCount <= delayCount ){

ovCheck = TIFR0 & 0x01;	//variable to check if overflow met.

if (ovCheck == 1){

ovFlowCount ++;		//increase overflow count

TIFR0 |= (1<<TOV0); //reset sreg flag

}

}

ovFlowCount = 0;

PORTB |= (1<<2);

flag = 0;

}

}

-----------------------------------------------------------------------------------------

//DA2A task1

#include <avr/io.h>

#include <stdio.h>

#include <avr/interrupt.h>

//declare vital variables

int matchCount = 0;

int dc60 = 23; //duty cycle of 60%

int dc40 = 15; //duty cycle of 40%

int flag = 1;  //flag used to toggle between duty cycles

int cycleMatch = 244;	//compare to 244 and clear each time

int main(void)

{

DDRB |= (1<<2)|(1<<5);	// Set all PORTB out

PORTB |= (1<<5);		//turn off LED 5

TCCR0A |= (1<<WGM01);	//set ctc mode on OCRA

TCCR0B |= (1<<CS00)|(1<<CS02);	// Set pre-scaler to 1024

TIMSK0 |= (1<<OCIE0A);	//enable compare interrupts

OCR0A = cycleMatch;

sei();					//enable interrupts

while(1);				//infinite loop

}

ISR(TIMER0_COMPA_vect){

matchCount ++;

if (flag == 1 && matchCount == dc60)//if the flag and the matchCount equals the cycle for 60%

{

PORTB &= ~(1<<2);	//toggle LED

matchCount = 0;	//reset overflow counter

flag = 0;		//set the flag for the second portion of the wave.

}

else if (flag == 0 && matchCount == dc40)//if the flag and the matchCount equals the cycles for 40%

{

PORTB |= (1<<2);	//toggle LED

matchCount = 0;	//reset overflow counter

flag = 1;

}

}

-----------------------------------------------------------------------------------------

//DA2A task2

#include <avr/io.h>

#include <stdio.h>

#include <avr/interrupt.h>

//set global variables

int flag = 0;

int matchCount = 0;

int cycleMatch = 244;

int main(void){

//initialize registers

DDRB |= (1<<2)|(1<<5);		//set portb 2 to output

PORTB |= (1<<2)|(1<<5);		//set pb2 to high

DDRC &= (0<<2);		//set ddrc to input

PORTC |= (0<<2);		//set portc2 to low

//Set modes

TCCR0A |= (1<<WGM01);	//set ctc mode on OCRA

TCCR0B |= (1<<CS00)|(1<<CS02);	// Set pre-scaler to 1024

TIMSK0 |= (1<<OCIE0A);	//enable compare interrupts

OCR0A = cycleMatch;

sei();			//enable interrupt

while (1) {

if (!(PINC & (1 << PINC2))) //check for button press

{

flag = 1;		//if pressed then turn on led

}

else {

PORTB |= (1<<2);		//if not pressed then keep led off

flag = 0;

}

}

return 0;

}

ISR(TIMER0_COMPA_vect){

int ovFlowCount = 0;

int matchCheck = 0;

int delayCount = 85;		//amount of overflows to reach desired delay

if (flag == 1)

{

flag = 0;	//toggle flag

PORTB &= ~(1<<2);

TCNT0 = 0;				//start counter at 0

while(matchCount <= delayCount )

{

matchCheck = TIFR0 & 0x02;	//variable to check if compare is met

if (matchCheck == 0x02){

matchCount ++;		//increase match count

TIFR0 |= (1<<OCF0A); //reset sreg flag

}

}

matchCount = 0;		//reset match count

PORTB |= (1<<2); //turn off LED

}

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://www.youtube.com/watch?v=mo6vX_DPnKQ

- GITHUB LINK OF THIS DA

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Andrew Buchanan