CPE301 – FALL 2019

# DA4B

Student Name: Andrew Buchanan

Student #:5003154346

Student Email: buchaa2@unlv.nevada.edu

Primary Github address: https://github.com/buchaa2/103EPC

Directory:

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

- #define F_CPU 16000000UL

- #include <avr/io.h>

- #include <avr/interrupt.h>

- #include <util/delay.h>

- volatile unsigned int ADCVal;

- void ADC_init();

- void timer_init();

- int main(void){

- // initialize ADC

- ADC_init();

- //set timer

- TCCR1B |=(1<<WGM12)|(1<<CS11); //CTC MODE prescaler = 8

- // set DDR's

- DDRB = 0xFF; //all outputs

- DDRC = 0;	//all input

- PORTB = 0;	//start at 0

- while(1){

- ADCSRA|=(1<<ADSC);

- while ((ADCSRA&(1<<ADIF))==0);	//wait until done converting

- ADCVal = ADC & 0x03FF;			//take only 10 bits

- OCR1A = 10*ADCVal;				//scale for proper delay

- //procedure for PB0

- PORTB |= (1<<PB0);

- while(!(TIFR1 & (1<<OCF1A))); //wait until timer flag is set. repeat

- TIFR1 |= (1<<OCF1A);

- PORTB &= ~(1<<PB0);

- while(!(TIFR1 & (1<<OCF1A)));

- TIFR1 |= (1<<OCF1A);

- //procedure for PB1

- PORTB |= (1<<PB1);

- while(!(TIFR1 & (1<<OCF1A)));

- TIFR1 |= (1<<OCF1A);

- PORTB &= ~(1<<PB1);

- while(!(TIFR1 & (1<<OCF1A)));

- TIFR1 |= (1<<OCF1A);

- //procedure for PB2

- PORTB |= (1<<PB2);

- while(!(TIFR1 & (1<<OCF1A)));

- TIFR1 |= (1<<OCF1A);

- PORTB &= ~(1<<PB2);

- while(!(TIFR1 & (1<<OCF1A)));

- TIFR1 |= (1<<OCF1A);

- //procedure for PB3

- PORTB |= (1<<PB3);

- while(!(TIFR1 & (1<<OCF1A)));

- TIFR1 |= (1<<OCF1A);

- PORTB &= ~(1<<PB3);

- while(!(TIFR1 & (1<<OCF1A)));

- TIFR1 |= (1<<OCF1A);

- }

- }

- void ADC_init(void){

- DIDR0 = 0x1;

- ADMUX = (1<<REFS0); //input is PC0

- ADCSRA |=(1<<ADEN)|(1<<ADPS2)|(1<<ADPS1)|(1<<ADPS0);

- ADCSRB = 0x0;

- }

Task 2

#define F_CPU 16000000L

#include <avr/io.h>

#include <util/delay.h>

volatile unsigned int ADCVal;

int main(void) {

//Set DDR's

DDRB = 0xFF; // Set port B as output

//Set ADC

DIDR0 = 0x1;

ADMUX = (1<<REFS0); //input pc0

ADCSRA |=(1<<ADEN)|(1<<ADPS2)|(1<<ADPS1)|(1<<ADPS0);

ADCSRB = 0x0;

TCCR1B = (1<<WGM13)|(1<<WGM12)|(1<<CS11)|(1<<CS10); //fast PWM mode and prescaler = 64

TCCR1A = (1<<COM1A1)|(1<<WGM11); // Non-inverting PWM

ICR1 = 2500; // Top of counter

//Main body loop

while (1)

{

ADCSRA |= (1 << ADSC); //start ADC conversion

while((!ADCSRA) &(1<<ADIF)); // Wait for conversion

OCR1A = ADC/2 ; //Set pwm with ADCValue

}

}

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

Task 1

https://www.youtube.com/watch?v=Hp5TtBw-bvM

task2

https://www.youtube.com/watch?v=oKQtA2IMzQ0

- GITHUB LINK OF THIS DA

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Andrew Buchanan