CPE301 – FALL 2019

# DA5

Student Name: Andrew Buchanan & Alira Coffman

Student #:5003154346

Student Email: buchaa2@unlv.nevada.edu

Primary Github address: https://github.com/buchaa2/103EPC

Directory:

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

#ifndef F_CPU

#define F_CPU 16000000UL

#endif

#include <avr/io.h>

#include <util/delay.h>

#include <avr/interrupt.h>

#include <stdbool.h>

#include <stdio.h>

#include <string.h>

#ifndef BAUD

#define BAUD 9600

#endif

#include "STDIO_UART.h"

#include "nrf24l01.h"

#include "nrf24l01-mnemonics.h"

#include "spi.h"

void print_config(void);

volatile bool message_received = false;

volatile bool status = false;

volatile uint8_t ADCdata;

volatile unsigned char temp[10];

int main(void)

{

ADMUX |= (1 << REFS0); // use AVcc

ADMUX |= (1 << ADLAR); // Right adjust

ADCSRA = (1 << ADEN)	// Enable

|(1 << ADPS1)

|(1 << ADPS0)	// 128 prescaler 16Mhz

|(1 << ADATE)	// ADC Auto Trigger

|(1 << ADSC);	// Start ADC

//	Set cliche message to send (message cannot exceed 32 characters)

char tx_message[32];				// Define string array

unsigned char i;

char dummy[10];

uart_init();

nrf24_init();

print_config();

nrf24_start_listening();

strcpy(tx_message,"GOOD");

nrf24_send_message(tx_message);

while (1)

{

ADCdata = (ADCH << 1) * 2 + 32; // Convert Celsius to Fahrenheit

itoa(ADCdata, dummy, 10); //convert char to ascii

for(i = 0 ; i < 10 ; i++)

{

temp[i] = dummy[i]; //move converted ascii

}

if (message_received)

{

//	Message received, print it

message_received = false;

printf("Received message: %s\n",nrf24_read_message());

//	Send message as response

_delay_ms(500);

status = nrf24_send_message(temp);

if (status == true) printf("Message sent successfully\n");

}

}

}

void print_config(void)

{

uint8_t data;

printf("Startup successful\n\n nRF24L01+ configured as:\n");

printf("-------------------------------------------\n");

nrf24_read(CONFIG,&data,1);

printf("CONFIG		0x%x\n",data);

nrf24_read(EN_AA,&data,1);

printf("EN_AA			0x%x\n",data);

nrf24_read(EN_RXADDR,&data,1);

printf("EN_RXADDR		0x%x\n",data);

nrf24_read(SETUP_RETR,&data,1);

printf("SETUP_RETR		0x%x\n",data);

nrf24_read(RF_CH,&data,1);

printf("RF_CH			0x%x\n",data);

nrf24_read(RF_SETUP,&data,1);

printf("RF_SETUP		0x%x\n",data);

nrf24_read(STATUS,&data,1);

printf("STATUS		0x%x\n",data);

nrf24_read(FEATURE,&data,1);

printf("FEATURE		0x%x\n",data);

printf("-------------------------------------------\n\n");

}

//	Interrupt on IRQ pin

ISR(INT0_vect)

{

message_received = true;

}

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

Task 1

https://www.youtube.com/watch?v=girBBwEW0g8

- GITHUB LINK OF THIS DA

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Andrew Buchanan